LOK JAGRUTI KENDRA UNIVERSITY

MCA, 2st Semester

Course Name: IOT (140119207)

NAME  :     Sahil Patel

ENROLLMENT NO :   24004401110312

DIV : I

ROLL NO : 06

INDEX

| Sr No | Page No | Name | Date | Sign |
| --- | --- | --- | --- | --- |
| 1 | 2 | Introduction to Arduino | 18/01/2025 |  |
| 2(A) | 4 | Create a circuit with code to flash an LED connected to an Arduino board. | 20/01/2025 |  |
| (B) | 5 | create a circuit with code to blink three LEDs connected to Arduino board. | 01/02/2025 |  |
| 3(A) | 6 | Create a circuit with code to flash an LED connected to an Arduino board. | 04/02/2025 |  |
| (B) | 8 | Create a circuit with code to flash an LED conneted to an Arduino board with breadboard. | 06/02/2025 |  |
| 4 | 9 | Create the circuit to connect the Arduino board to an RGB LED that blink. | 07/02/2025 |  |
| 5(A) | 10 | Design the circuit using a push button.activate and deactivate the LED attached the Arduino board. | 07/02/2025 |  |
| (B) | 12 | Design the circuit using a push button.activate and deactivate the LED attached the Arduino board with breadboard. | 08/02/2025 |  |
| 6 | 13 | Design circuit using IR sensor with Arduino and control LED by IR sensor. | 08/02/2025 |  |
| 7(A) | 17 | Design motion(PIR)Sensor. | 08/02/2025 |  |
| (B) | 18 | Design Circuit using PIR Sensor . Activate and Deactivate the LED attached to the Arduino board. | 08/02/2025 |  |
| (C) | 19 | Design circuit using PIR sensor . Activate and deactivate the buzzer attached to the ardiuno board. | 08/02/2025 |  |
| 8 | 21 | Design circuit using ultrasonic sensor for distance measurement. | 08/02/2025 |  |
| 9 | 23 | Implement the circuit gas sensor for smoke detection. | 17/02/2025 |  |
| 10 | 24 | Design Soil Moisture sensor using Arduino board and result display on LCD. | 20/02/2025 |  |

Submitted By: Sahil Patel (24004401110312) 		Submitted To: Ms. Neha Namdev

Introduction to Arduino

- Object: Introduction to Arduino

- Components: Arduino UNO R3

- Theory:

  - Power Section

- 7 to 12V DC Input: External power supply input. Accepts 2.1mm x 5.5mm DC barrel jack (center positive).

- Voltage Regulator: Converts input voltage to a regulated 5V for the board.

- 16MHz Crystal Oscillator: Provides the clock signal to the microcontroller for timing.

  - Power Pins

- Reset Pin: Resets the microcontroller.

- 3.3V Output: Provides 3.3V for powering external components.

- 5V Output: Provides 5V for powering external components.

- GND: Ground pins for circuit connection.

- Vin: Input voltage to the board when using an external power source.

- Analog Pins

- Analog Pins A0 to A5: These are the analog input pins. They convert analog signals to digital values using the ADC (10-bit resolution).

- Microcontroller

- ATmega328: The main microcontroller IC on the board, which executes the code uploaded by the user.

- Digital I/O Pins

- Digital Pins 0 to 13: Used for general-purpose I/O. Some of these have special functions:

- Pin 13 LED: Built-in LED connected to pin 13.

- PWM Pins (~): Pins capable of Pulse Width Modulation (3, 5, 6, 9, 10, 11).

- SPI Pins: Pins for Serial Peripheral Interface (10: SS, 11: MOSI, 12: MISO, 13: SCK).

- Communication Pins

- (I2C) SDA and SCL: Serial Data and Serial Clock pins for I2C communication.

- TXD and RXD: Transmit and Receive pins for UART communication.

- Programming and USB Interface

- ATmega16U2 IC: Microcontroller used as the USB-to-serial converter.

- USB-B Port: Used to connect the board to a computer for power, programming, and serial communication.

- ICSP Headers

- ICSP for ATmega328: In-Circuit Serial Programming pins to program the microcontroller.

- ICSP for USB Interface: For reprogramming the ATmega16U2 chip.

- Other Features

- Reset Button: Manually resets the microcontroller.

- AREF Pin: Analog Reference Voltage for the ADC. Used to set an external reference voltage for analog inputs.

- Arduino Uno R3 Programming

- The programming of an Arduino Uno R3 can be done using IDE software. The microcontroller on the board will come with pre-burned by a boot loader that permits to upload fresh code without using an exterior hardware programmer.

- The communication of this can be done using a protocol like STK500.

- We can also upload the program in the microcontroller by avoiding the boot loader using the header like the In-Circuit Serial Programmin

2.A) Create a circuit with code to flash an LED connected to an Arduino board.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| D1 | 1 | Red LED |
| R1 | 1 | 220 Ω Resistor |

Theory:

Arduino Circuit: An Arduino reads inputs (like sensor data or button presses) and controls outputs (like motors or LEDs).

LED Light: LED (Light Emitting Diode) produces light when current flows through it.

Register: Stores control settings, data, and status information for a microcontroller.

INPUT                                                                                    OUTPUT

// C++ code

//

void setup()

{

pinMode(13, OUTPUT);

}

void loop()

{

digitalWrite(13, HIGH);

delay(500); // Wait for 1000 millisecond(s)

digitalWrite(13, LOW);

delay(500); // Wait for 1000 millisecond(s)

}

2.B) create a circuit with code to blink three LEDs connected to Arduino board.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| D1 | 1 | Red LED |
| D2 | 1 | Green LED |
| D3 | 1 | Orange LED |
| R1 | 1 | 220 Ω Resistor |

Theory:

Arduino Circuit: Reads inputs (sensor, button) and controls outputs (motor, LED).

LED Light: Stands for Light Emitting Diode.

Register: Manages control, data, and status in a microcontroller.

INPUT                                                                                OUTPUT

// C++ code

//

void setup()

{

pinMode(12, OUTPUT);

pinMode(13, OUTPUT);

pinMode(11, OUTPUT);

}

void loop()

{

digitalWrite(13, HIGH);

digitalWrite(12, LOW);// Wait for 1000 millisecond(s)

digitalWrite(11, LOW);

delay(500);

digitalWrite(13, LOW);

digitalWrite(12, HIGH);// Wait for 1000 millisecond(s)

digitalWrite(11, LOW);

delay(500);

digitalWrite(13, LOW);

digitalWrite(12, LOW);// Wait for 1000 millisecond(s)

digitalWrite(11, HIGH);

delay(500);

}

3.A) Create a circuit with code to flash an LED connected to an Arduino board.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| R1 | 1 | 220 Ω Resistor |
| D1 | 1 | Red LED |
| B1 | 1 | Breadboard Small |

Theory:

Arduino Circuit: Reads inputs (sensor, button) and controls outputs (motor, LED).

LED Light: Stands for Light Emitting Diode.

Register: Manages control, data, and status in a microcontroller.

Breadboard Small: A plastic board with holes for building circuits without soldering. It holds wires, components, and ICs, with internally connected rows for easy prototyping.

INPUT						            	 OUTPUT

// C++ code

//

int pin = 0;

int set = 0;

void setup()

{

pinMode(13, OUTPUT);

pinMode(12, OUTPUT);

pinMode(11, OUTPUT);

}

void loop()

{

pin = 0;

digitalWrite(13, HIGH);

digitalWrite(12, LOW);

digitalWrite(11, LOW);

delay(100); // Wait for 100 millisecond(s)

digitalWrite(13, HIGH);

digitalWrite(12, HIGH);

digitalWrite(11, LOW);

delay(100); // Wait for 100 millisecond(s)

digitalWrite(13, LOW);

digitalWrite(12, LOW);

digitalWrite(11, HIGH);

delay(100); // Wait for 100 millisecond(s)

}

3.B) Create a circuit with code to flash an LED conneted to an Arduino board with breadboard.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| D1 | 1 | Orange LED |
| D2 | 1 | Green LED |
| D3 | 1 | Blue LED |
| R1 | 1 | 220 Ω Resistor |
| B1 | 1 | Breadboard Small |

Theory:

Arduino Circuit: Reads inputs and controls outputs.

LED: Light Emitting Diode.

Register: Handles control, data, and status.

Breadboard: Solder-free circuit board.

INPUT							             OUTPUT

// C++ codE

int set = 0;

void setup()

{

pinMode(13, OUTPUT);

pinMode(12, OUTPUT);

pinMode(11, OUTPUT);

}

void loop()

{

set = 0;

digitalWrite(13, HIGH);

digitalWrite(12, LOW);

digitalWrite(11, LOW);

delay(500); // Wait for 500 millisecond(s)

digitalWrite(13, LOW);

digitalWrite(12, HIGH);

digitalWrite(11, LOW);

delay(500); // Wait for 500 millisecond(s)

digitalWrite(13, LOW);

digitalWrite(12, LOW);

digitalWrite(11, HIGH);

delay(500); // Wait for 500 millisecond(s)

}

4.Create the circuit to connect the Arduino board to an RGB LED that blink.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| R1 | 1 | 220 Ω Resistor |
| D1 | 1 | RCBG LED RGB |

Theory:

Arduino Uno: Microcontroller board for LED control.

RGB LED: Emits red, green, and blue light.

Resistor: Limits current, usually 220Ω.

Input  							         Output

// C++ code

//

int pin = 0;

int set = 0;

void setup()

{

pinMode(13, OUTPUT);

pinMode(12, OUTPUT);

pinMode(11, OUTPUT);

}

void loop()

{

pin = 0;

digitalWrite(13, HIGH);

digitalWrite(12, LOW);

digitalWrite(11, LOW);

delay(100); // Wait for 100 millisecond(s)

digitalWrite(13, HIGH);

digitalWrite(12, HIGH);

digitalWrite(11, LOW);

delay(100); // Wait for 100 millisecond(s)

digitalWrite(13, LOW);

digitalWrite(12, LOW);

digitalWrite(11, HIGH);

delay(100); // Wait for 100 millisecond(s)

}

5.A)  Design the circuit using a push button.activate and deactivate the LED attached the Arduino board.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| S2 | 1 | Pushbutton |
| D2 | 1 | Red LED |
| R1, R2 | 2 | 220 Ω Resistor |

Theory:

When pressed, the button reads HIGH; when released, it reads LOW (via pull-down resistor). Arduino toggles the LED based on this input.

Input                                                       Output

// C++ code

int button = 0;

void setup()

{

pinMode(2, INPUT);

Serial.begin(9600);

pinMode(11, OUTPUT);

}

void loop()

{

button = digitalRead(2);

Serial.println(button);

if (button == 1) {

digitalWrite(11, HIGH);

} else {

digitalWrite(11, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

5.B)  Design the circuit using a push button.activate and deactivate the LED attached the Arduino board with breadboard.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| S2 | 1 | Pushbutton |
| D2 | 1 | Red LED |
| R1, R2 | 2 | 220 Ω Resistor |
| B1 | 1 | Breadboard Small |

Theory:

Pressing the push button (HIGH) turns the LED on; releasing it (LOW) turns it off using a pull-down resistor on a breadboard.

Input							   Output

// C++ code

//

int push = 0;

void setup()

{

pinMode(2, INPUT);

Serial.begin(9600);

pinMode(10, OUTPUT);

}

void loop()

{

push = digitalRead(2);

Serial.println(push);

if (push == 1) {

digitalWrite(10, HIGH);

} else {

digitalWrite(10, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

6.Design circuit using IR sensor  with Arduino and control LED by IR senso r.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| D1 | 1 | Red LED |
| U2 | 1 | IR sensor |
| R1 | 1 | 220 Ω Resistor |

Theory:

The Arduino reads the IR sensor's output. When the sensor detects an object, it sends a HIGH signal, turning the LED on. When no object is detected (LOW), the LED turns off. A 220Ω resistor limits the LED current.

Input                                                            Output

// C++ code

//

#include <IRremote.h>

int button = 0;

// Map the IR code to the corresponding remote button.

// The buttons are in this order on the remote:

//    0   1   2

//    4   5   6

//    8   9  10

//   12  13  14

//   16  17  18

//   20  21  22

//   24  25  26

//

// Return -1, if supplied code does not map to a key.

int mapCodeToButton(unsigned long code) {

// For the remote used in the Tinkercad simulator,

// the buttons are encoded such that the hex code

// received is of the format: 0xiivvBF00

// Where the vv is the button value, and ii is

// the bit-inverse of vv.

// For example, the power button is 0xFF00BF000

// Check for codes from this specific remote

if ((code & 0x0000FFFF) == 0x0000BF00) {

// No longer need the lower 16 bits. Shift the code by 16

// to make the rest easier.

code >>= 16;

// Check that the value and inverse bytes are complementary.

if (((code >> 8) ^ (code & 0x00FF)) == 0x00FF) {

return code & 0xFF;

}

}

return -1;

}

int readInfrared() {

int result = -1;

// Check if we've received a new code

if (IrReceiver.decode()) {

// Get the infrared code

unsigned long code = IrReceiver.decodedIRData.decodedRawData;

// Map it to a specific button on the remote

result = mapCodeToButton(code);

// Enable receiving of the next value

IrReceiver.resume();

}

return result;

}

void setup()

{

IrReceiver.begin(2);

Serial.begin(9600);

pinMode(13, OUTPUT);

}

void loop()

{

button = readInfrared();

Serial.println(button);

delay(100); // Wait for 100 millisecond(s)

if (button == 16) {

digitalWrite(13, HIGH);

}

if (button == 17) {

digitalWrite(13, LOW);

}

delay(100); // Wait for 100 millisecond(s)

delay(1000); // Wait for 1000 millisecond(s)

delay(100); // Wait for 100 millisecond(s)

}

7.A) Design motion(PIR)Sensor.

Theory:

A PIR (Passive Infrared) sensor detects motion by sensing infrared radiation changes. When motion is detected, it outputs HIGH, turning the LED on. If no motion is detected, it outputs LOW, turning the LED off. The Arduino processes this signal to control the LED.

Design

7.B) Design Circuit using PIR Sensor . Activate and Deactivate the LED attached to the Arduino board.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| PIRSensor | 1 | PIR Sensor |
| D1 | 1 | Red LED |
| R1 | 1 | 220 Ω Resistor |

Theory:

The PIR sensor detects motion and sends a HIGH signal to the Arduino, turning the LED on. When no motion is detected (LOW), the LED turns off. A 220Ω resistor limits the LED current.

Input							    Output

// C++ code

//

int input = 0;

void setup()

{

pinMode(12, INPUT);

Serial.begin(9600);

pinMode(13, OUTPUT);

}

void loop()

{

input = digitalRead(12);

Serial.println(input);

if (input == 1) {

digitalWrite(13, HIGH);

} else {

digitalWrite(13, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

7.C) Design circuit using PIR sensor . Activate and deactivate the buzzer attached to the ardiuno board.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| PIRSensor | 1 | PIR Sensor |
| R1 | 1 | 220 Ω Resistor |
| PIEZO1 | 1 | Piezo |
| D1 | 1 | Red LED |

Theory:

The PIR sensor detects motion and sends a HIGH signal to the Arduino, activating the buzzer. When no motion is detected (LOW), the buzzer turns off. A 220Ω resistor limits the current.

Input						     Output

// C++ code

//

int input = 0;

void setup()

{

pinMode(12, INPUT);

Serial.begin(9600);

pinMode(13, OUTPUT);

}

void loop()

{

input = digitalRead(12);

Serial.println(input);

if (input == 1) {

digitalWrite(13, HIGH);

} else {

digitalWrite(13, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

8. A) Design circuit using ultrasonic sensor for distance measurement.

- .A. Design Circuit using Ultrasonic  sensor for distance measurment.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| DIST1 | 1 | Ultrasonic Distance Sensor (4-pin) |
| U1 | 1 | Arduino Uno R3 |

Theory:

The ultrasonic sensor measures distance by sending and receiving sound waves. The Arduino calculates the time taken for the echo to return and converts it into distance.

Input					               Output

// C++ code

//

int input = 0;

long readUltrasonicDistance(int triggerPin, int echoPin)

{

pinMode(triggerPin, OUTPUT);  // Clear the trigger

digitalWrite(triggerPin, LOW);

delayMicroseconds(2);

// Sets the trigger pin to HIGH state for 10 microseconds

digitalWrite(triggerPin, HIGH);

delayMicroseconds(10);

digitalWrite(triggerPin, LOW);

pinMode(echoPin, INPUT);

// Reads the echo pin, and returns the sound wave travel time in microseconds

return pulseIn(echoPin, HIGH);

}

void setup()

{

Serial.begin(9600);

pinMode(13, OUTPUT);

}

void loop()

{

input = 0.01723 * readUltrasonicDistance(9, 8);

Serial.println(input);

if (input > 150) {

digitalWrite(13, HIGH);

} else {

digitalWrite(13, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

9. Implement the circuit gas sensor for smoke detection.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| GAS1 | 1 | Gas Sensor |
| R1, R2 | 2 | 220 Ω Resistor |
| D1 | 1 | Red LED |

Theory:

The gas sensor detects smoke and sends a HIGH signal to the Arduino, turning the LED on as an alert. When no smoke is detected (LOW), the LED remains off. Resistors help regulate the circuit.

Input 							     Output

// C++ code

//

int gas = 0;

void setup()

{

pinMode(A0, INPUT);

Serial.begin(9600);

pinMode(13, OUTPUT);

pinMode(13, OUTPUT);

}

void loop()

{

gas = analogRead(A0);

Serial.println(gas);

if (gas > 200) {

tone(13, 2700309, 8000); // play tone 208 (E17 = 2700309 Hz)

} else {

digitalWrite(13, LOW);

}

delay(10); // Delay a little bit to improve simulation performance

}

10) . Design Soil Moisture sensor using Arduino board and result display on LCD.

Components:

| Name | Quantity | Component |
| --- | --- | --- |
| U1 | 1 | Arduino Uno R3 |
| U2 | 1 | PCF8574-based, 39 (0x27) LCD 16 x 2 (I2C) |
| SEN1 | 1 | Soil Moisture Sensor |

Theory:

The soil moisture sensor measures soil humidity and sends data to the Arduino. The moisture level is displayed on the 16x2 LCD via the I2C module. Higher moisture gives a LOW reading, while dry soil gives a HIGH reading.

Input:

Code:

// C++ code

#include <LiquidCrystal_I2C.h> int input = 0;

LiquidCrystal_I2C lcd_1(39, 16, 2); void setup()

{

lcd_1.init();

pinMode(A5, INPUT);

Serial.begin(9600);

}

void loop()

{

input = analogRead(A5);   Serial.println(input);   lcd_1.clear();   lcd_1.backlight();   lcd_1.setCursor(0, 0);   lcd_1.print("Soil Sensor ");   lcd_1.print(input);   delay(3000); // Wait for 3000 millisecond(s)

}

Output:

Submitted to:

Ms. Neha Namdev

Submitted by:  Sahil Patel

Date:  19-01-2025